Step 1: Setup Requirements

1 Router

1 Switch

2 PCs

Step 2: Physical Connections

Attacker PC Router (Fa0/1)

Router (Fa0/0)Switch

SwitchInternal PC

Step 3: IP Configuration

Device	             IP Address	Subnet Mask	Gateway

Attacker PC	10.0.0.2	255.255.255.0	10.0.0.1

Internal PC	192.168.1.2	255.255.255.0	192.168.1.1

Step 4: Router Configuration (CLI)

Router  CLI

enable

configure terminal

interface GigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

Interface GigabitEthernet 0/1

ip address 10.0.0.1 255.255.255.0

no shutdown

exit

Step 5: Create IDS

Router  CLI

access-list 101 deny tcp any any eq 23(telnet)

access-list 101 deny tcp any any eq 21(ftp)

access-list 101 permit ip any any

Step 6: Apply ACL to Router

Interface GigabitEthernet 0/1

ip access-group 101 in

exit

Step 7: Save config

end

write memory

Step 8: Test Project

Attacker PC  Command Prompt

telnet 192.168.1.2

[Connection should fail (blocked)]

ping 192.168.1.2

[Should work]

Step 9: Check IDS alert

RouterCLI

show access-list

[deny tcp any any eq 23 (5 matches)]

Step 10: Created web based IDS dashboard

Show access-list

Developed a web app using Python (Flask) to display:

- Network traffic

- Security alerts

- Blocked attacks

Step 11: Integrated python program

Created a file app.py

This program:

- Runs a web server

- Receives/logs network events

- Displays them in browser

Step 12: Event processing logic

Inside the program, we implemented logic to classify traffic:

Traffic type	   Condition	        Action

HTTP (Port 80)	   Normal traffic	        Allowed (traffic)

FTP (Port 21)	   Suspicious	        Alert

Telnet (Port 23)	   Attack	                      Alert + Block

## Step 13: Dashboard Features

The web page displays:

### 📊 1. Traffic Monitoring

- Shows allowed traffic like HTTP

### ⚠️ 2. Alerts

- Shows suspicious activity like FTP

### 🚫 3. Blocked Attacks

- Shows blocked Telnet attacks using ACL

### 📈 4. Counters

- Total traffic count

- Total alerts

- Total blocks

### 🔴 5. Live Status

- SAFE / ATTACK DETECTED

## Step 14: Real-Time Display

The dashboard:

- Updates automatically every few seconds

- Displays latest network activity

- Acts like a basic IDS monitoring system

## Step 15: Testing with Web Dashboard

### Test 1: Normal Traffic

ping 192.168.1.2

Result:

- Allowed

- Shown as Traffic in dashboard

### Test 2: Attack (Telnet)

telnet 192.168.1.2

👉 Result:

- Blocked by ACL

- Shown as:

  - 🚨 Alert

  - ❌ Block

### Test 3: FTP Access

ftp 192.168.1.2

Result:

- Detected as suspicious

- Shown as Alert